FZ-1/5498/AJ/21										Annex No. 1

Technical requirements:

The subject of the preliminary inquiry is the purchase of laboratory extruder – 1 pcs.

The offered device must be brand new and must to meet the following requirements:

1. Laboratory extruder has to have following parameters:

- max. dimension: 21″ x 17″ x 23″ (L x W x H),

- working temperature: 0-150°C,

- max. working pressure: 4000 psi (276 bar),

- screw size – 1″ (25.4mm), 6:1 L/D,

- power – 1/2HP (0.37 kW), 760 in-lb (85.9 nm),

- max. current – 1.9 Amps (230 V to motor),

- voltage – 120 V, 1 input, 230 V, 3 to motor,

- frequency – 50 Hz input, 60 Hz output,

- max. screw speed – 41 rpm,

- volumetric capacity of screw – 4.13 in3 (67.7 cc),

- min. batch size – 140 g,

- twin packer feeding system,

- the manufacturer must have a quality management system (CE).

2. Laboratory extruder should be equipped with:

- temperature read out (Celsius or Fahrenheit),

- pressure read out (PSI or BAR),

- rotary die face cutter, replaceable cutter blades configurable to 1, 2 and 4 cuts per turn, speed adjustable,

- water heating or cooling,

- temperature control unit,

- 1 die (1/8” dia,  the dies are wearable items),

- emergency stop button.