FZ-1/5544/AJ/21								Annex No. 1 to the inquiry

Technical requirements:

The subject of the inquiry is the purchase of chemical igniters of energy 5 kJ (750 pcs.) and chemical igniters of energy 1 kJ (750 pcs.).

The offered chemical igniters must be brand new, adapted to use for testing the explosive properties of dusts in owned by the Buyer a 20-liter spherical chamber (research apparatus KD-2 - GIG). The offered chemical igniters

must be approved for use in the EU. The offered chemical igniters must to meet the following requirements:

| Description | Quantity pcs. |
| --- | --- |
| Chemical igniters of energy 5 kJ class 1. The composition of the pyrotechnic material should be the following: - 40 wt % zirconium - 30 wt % barium nitrate - 30 wt % barium peroxide The pyrotechnic charge is in a plastic shell, which is placed in a 25mm long aluminum casing. Whole is closed with a flexible cap. Permissible operating temperature: - 25 oC to + 50 oC No - Fire current: 0,18 A All - Fire current: 0,6 A Total resistance 1,10 – 2,20 Ω (wire length 20 cm) | 750 |
| Chemical igniters of energy 1 kJ class 1. The composition of the pyrotechnic material should be the following: - 40 wt % zirconium - 30 wt % barium nitrate - 30 wt % barium peroxide The pyrotechnic charge is in a plastic shell, which is placed in a 25mm long aluminum casing. Whole is closed with a flexible cap. Permissible operating temperature: - 25 oC to + 50 oC No - Fire current: 0,18 A All - Fire current: 0,6 A Total resistance 1,10 – 2,20 Ω (wire length 20 cm) | 750 |