FZ/5650/AJ/22										Annex No. 1

Technical requirements:

The subject of the preliminary inquiry is the purchase of laboratory extruder – 1 pcs.

The offered device must be brand new and must to meet the following requirements:

1. Laboratory extruder has to have following parameters:

- max. dimension: 600 mm x 500 mm x 600 mm  (L x W x H),

- working temperature: 0-150°C,

- max. working pressure: 4000 psi (276 bar),

- screw size – 1″ (25.4mm), 6:1 L/D,

- power – min. 0.35 kW,

- frequency – 50-60 Hz input, 60 Hz output,

- screw speed range – 5-40 rpm,

- min. volumetric capacity of screw – 65 cm3

- min. batch size – 140 g,

- twin packer feeding system,

- extruder elements should be made of steel  min. 304 SS (Hopper), 17-4PH SS (Auger and Packers),

- the manufacturer must have a quality management system (CE).

2. Laboratory extruder should be equipped with:

- temperature read out (Celsius or Fahrenheit),

- pressure read out (PSI or BAR),

- rotary die face cutter, replaceable cutter blades configurable to 1, 2 and 4 cuts per turn, speed adjustable,

- water heating or cooling,

- temperature control unit,

- 1 die (1/8” dia,  the dies are wearable items),

- emergency stop button.

The extruder is designed to test the following materials: ash from coal combustion, ash from biomass combustion, biomass, fine coal, coal sludge.